120″ Telescope Coude′ Focus

| Optic # | Optic Notes | Procedures |
| --- | --- | --- |
| #1 | Bare aluminum Subjected to oil drips and occasional moisture Up facing mirror – Protective cover | Inspect monthly Co2 monthly Wash annually |
| #2 | Bare aluminum Bad glass – coating degrades within 6 months Down facing mirror – Protective cover | Inspect monthly Carefully wash as needed |
| #3 | SiO overcoated aluminum Subjected to oil drips and occasional moisture Up facing tertiary mirror – Protective cover | Inspect monthly Co2 every other month Wash as needed |
| #4 | SiO overcoated aluminum Down facing arch mirror – Protective cover | Inspect every 6 month Wash as needed |
| #5 | SiO overcoated aluminum Subjected to much dust and dirt Up facing mirror in fork polar opening – Protective cover | Inspect every 6 month Co2 every other month Wash as needed |